1.0 BACKGROUND

Heavy metals are defined as metallic elements that have a relatively high density compared to water. With the assumption that heaviness and toxicity are inter-related, heavy metals also include metalloids, such as arsenic, that are able to induce toxicity at low level of exposure. In recent years, there has been an increasing ecological and global public health concern associated with environmental contamination by these metals. Also, human exposure has risen dramatically as a result of an exponential increase of their use in several industrial, agricultural, domestic and technological applications. Reported sources of heavy metals in the environment include geogenic, industrial, agricultural, pharmaceutical, domestic effluents, and atmospheric sources. Environmental pollution is very prominent in point source areas such as mining, foundries and smelters, and other metal-based industrial operations (Ogwueleka, 2003). Although heavy metals are naturally occurring elements that are found throughout the earth’s crust, most environmental contamination and human exposure result from anthropogenic activities such as mining and smelting operations, industrial production and use, and domestic and agricultural use of metals and metal-containing compounds. Environmental contamination can also occur through metal corrosion, atmospheric deposition, soil erosion of metal ions and leaching of heavy metals, sediment re-suspension and metal evaporation from water resources to soil and ground water. Natural phenomena such as weathering and volcanic eruptions have

also been reported to significantly contribute to heavy metal pollution. Industrial sources include metal processing in refineries, coal burning in power plants, petroleum combustion, nuclear power stations and high tension lines, plastics, textiles, microelectronics, wood preservation and paper processing plants (Dara, 1993).

It has been reported that metals such as cobalt (Co), copper (Cu), chromium (Cr), iron (Fe), magnesium (Mg), manganese (Mn), molybdenum (Mo), nickel (Ni), selenium (Se) and zinc (Zn) are essential nutrients that are required for various biochemical and physiological functions. Inadequate supply of these micro-nutrients results in a variety of deficiency diseases or syndromes (Nwajei et al., 2007) .

Heavy metals are also considered as trace elements because of their presence in trace concentrations (ppb range to less than 10ppm) in various environmental matrices]. Their bioavailability is influenced by physical factors such as temperature, phase association, adsorption and sequestration. It is also affected by chemical factors that influence speciation at thermodynamic equilibrium, complexation kinetics, lipid solubility and octanol/water partition coefficients. Biological factors such as species characteristics, trophic interactions, and biochemical/physiological adaptation, also play an important role (Dara, 1993).

The essential heavy metals exert biochemical and physiological functions in plants and animals. They are important constituents of several key enzymes and play important roles in various oxidation-reduction reactions. Copper for example serves as an essential co-factor for several oxidative stress-related enzymes including catalase, superoxide dismutase, peroxidase, cytochrome c oxidases, ferroxidases, monoamine oxidase, and dopamine β-monooxygenase (Osuj & Onojake, 2004). Hence, it is an essential nutrient that is incorporated into a number of metalloenzymes involved in hemoglobin formation, carbohydrate metabolism, catecholamine biosynthesis, and cross-linking of collagen, elastin, and hair keratin. The ability of copper to cycle between an oxidized state, Cu(II), and reduced state, Cu(I), is used by cuproenzymes involved in redox reactions. However, it is this property of copper that also makes it potentially toxic because the transitions between Cu(II) and Cu(I) can result in the generation of superoxide and hydroxyl radicals. Also, excessive exposure to copper has been linked to cellular damage leading to Wilson disease in humans. Similar to copper, several other essential elements are required for biologic functioning, however, an excess amount of such metals produces cellular and tissue damage leading to a variety of adverse effects and human diseases. For some including chromium and copper, there is a very narrow range of concentrations between beneficial and toxic effects. Other metals such as aluminium (Al), antinomy (Sb), arsenic (As), barium (Ba), beryllium (Be), bismuth (Bi), cadmium (Cd), gallium (Ga), germanium (Ge), gold (Au), indium (In), lead (Pb), lithium (Li), mercury (Hg), nickel (Ni), platinum (Pt), silver (Ag), strontium (Sr), tellurium (Te), thallium (Tl), tin (Sn), titanium (Ti), vanadium (V) and uranium (U) have no established biological functions and are considered as non-essential metals (Fytianos etal., 2001).

In biological systems, heavy metals have been reported to affect cellular organelles and components such as cell membrane, mitochondrial, lysosome, endoplasmic reticulum, nuclei, and some enzymes involved in metabolism, detoxification, and damage repair. Metal ions have been found to interact with cell components such as DNA and nuclear proteins, causing DNA damage and conformational changes that may lead to cell cycle modulation, carcinogenesis or apoptosis. Several studies from our laboratory have demonstrated that reactive oxygen species (ROS) production and oxidative stress play a key role in the toxicity and carcinogenicity of metals such as arsenic, cadmium, chromium, lead, and mercury. Because of their high degree of toxicity, these five elements rank among the priority metals that are of great public health significance. They are all systemic toxicants that are known to induce multiple organ damage, even at lower levels of exposure. According to the United States Environmental Protection Agency (U.S. EPA), and the International Agency for Research on Cancer (IARC), these metals are also classified as either “known” or “probable” human carcinogens based on epidemiological and experimental studies showing an association between exposure and cancer incidence in humans and animals (Fytianos etal., 2001).

Heavy metal-induced toxicity and carcinogenicity involves many mechanistic aspects, some of which are not clearly elucidated or understood. However, each metal is known to have unique features and physic-chemical properties that confer to its specific toxicological mechanisms of action. This review provides an analysis of the environmental occurrence, production and use, potential for human exposure, and molecular mechanisms of toxicity, genotoxicity, and carcinogenicity of arsenic, cadmium, chromium, lead, and mercury (Ebong et al., 2008).

1.1 EFFECTS OF SOME HEAVY METALS

Heavy metals are toxic to soil, plants, aquatic life and human health if their concentration is high in the compost. Heavy metals exhibit toxic effects towards soil biota by affecting key microbial processes and decrease the number and activity of soil microorganisms. Even low concentration of heavy metals may inhibit the physiological metabolism of plant. Uptake of heavy metals by plants and subsequent accumulation along the food chain is a potential threat to animal and human health. Contaminants in aquatic systems, including heavy metals, stimulate the production of reactive oxygen species (ROS) that can damage fishes and other aquatic organisms. Hence the compost has to be use for agriculture it should be free from heavy metals. Therefore, the present study evaluated the effects of heavy metal containing compost on soil, plants, human health and aquatic life (Ebong et al., 2008).

1.2 SOURCES OF HEAVY METALS

Heavy metals are naturally present in our environment. They are present in the atmosphere, lithosphere, hydrosphere and biosphere. Although these heavy metals are present in the ecosystem, their exposure to humans is through various anthropogenic activities of man. In the earth crust, these heavy metals are present in ores which are recovered during mining activities as minerals. In most ores heavy metals such as arsenic, iron, lead, zinc, gold, nickel, silver and cobalt exist as sulfides while others such as manganese, aluminum, selenium gold, and antimony exist as oxides. Certain heavy metals such as copper, iron and cobalt can exist both as sulfide and oxide ores. Some sulfides may contain two or more heavy metals together such as chalcopyrite, (CuFeS2) which contains both copper and iron. During these mining activities, heavy metals are released from the ore and scattered in open in the environment; left in the soil, transported by air and water to other areas. Furthermore, when these heavy metals are used in the industries for various industrial purposes, some of these elements are released into the air during combustion or into the soil or water bodies as effluents. More so, the industrial products such as paints, cosmetics, pesticides, and herbicides also serve as sources of heavy metals. Heavy metals may be transported through erosion, run-off or acid rain to different locations on soils and water bodies (Ebong et al., 2008).

1.3 SOME SPECIFIC HEAVY METALS EFFECTS

1.3.1 CADMIUM (Cd)

Cadmium is a white shining but tarnish able metals similar in several characteristics to zinc and tin. Cadmium is not found to a great extend in nature its presence in the earth crust is estimated to a range between 0.15 to 0.11mg/g (67 elements in order of abundance), with a Zn/Cd ratio around 250:1 (the ratio depends strictly on the nature of rocks) (Nwajei et al., 2007).

Cadmium can be present as a results of volcanic emission and released from vegetation. It is not essential to plants growth, but under certain condition can accumulates in some plant to level that are hazardous to animal and human. Some sewage slugs contain enough cadmium to encourage accumulation. The chemistry of cadmium reaction in the soil is not well understood, but it is known that the uptake of these element is generally reduced by organic matter, silicate clay and hydro oxide or iron and aluminium when the soil is aerated. Cadmium uptake is high in excess. Soil and is reduced when soil is limits. This element has a wavelength of 228mm. it is a hazardous air pollutant which enter plants and animals from soil and water, thus cadmium has inhibitory effects on antioxidant processes, its interact with sulphide group of essential enzymes. Chronic exposure to cadmium is associated with a wide range of disease, including heart diseases, anaemia, skeletal weaken, depressed immune system responses. At extreme level it causes an illness called tai-‘’ital’’ disease characteristics by  brittle bones and intense pain (Connell and miller 1984 )

1.3.2 CHROMIUM (Cr)

The most common forms of chromium are chromium (VI) and chromium (III) (Hilgenkamp, 2006). Although chromium toxicity in the environment is rare. It still presents some risks to human health since chromium can be accumulated on skin, lungs, muscle, fat, in liver, dorsal spin, hair, nails and placenta where it is traceable to various health conditions (Adeleken and Abegunde, 2011). The health effects brought about by the exposure to chromium (VI) include lung cancer, malignant neoplasia, chromium dermatitis and skin ulcers (Sarkar, 2005). Perforations and ulcerations of the nasal septum and bronchial asthma have also been reported. In one of the studies, a four-fold increase in childhood leukemia was attributed to possible consumption of water with chromium (VI) levels above standard recommended value of 300 mg/kg (Sarkar, 2005). The sources of chromium in the environment include, cement, leather, plastics, dyes, textiles, paints, printing ink, cutting oils, photographic materials, detergents, wood preservatives among others (Hilgenkamp, 2006). Other sources of chromium are power plants, liquid fuels, brown and hard coal and industrial and municipal wastes. Non biodegradability of chromium is responsible for its persistence in the environment and once mixed with soil, it undergoes transformation into various mobile forms before ending into environmental sink (Adeleken and Abegunde, 2011). The maximum allowable limit of chromium in the soil set by United Kingdom is 300 mg/kg (EU, 1986) while the limit set by FEPA (1991) is 0.03 mg/kg in Nigeria.

1.3.3 COPPER (Cu)Copper is an essential nutrient that is incorporated into a number of metalloenzymes involved in hemoglobin formation, drug/xenobiotic metabolism, carbohydrate metabolism, catecholamine biosynthesis, the crosslinking of collagen, elastin and hair keratin, and the antioxidant defense mechanism. Copper-dependent enzymes, such as cytochrome C oxidase, superoxide dismutase, ferroxidases, monoamine oxidase and dopamine β-monooxygenase, function to reduce activated oxygen species or molecular oxygen. Symptoms associated with copper deficiency in humans include normocytic, hypochromic anaemia, leukopenia, and osteoporosis (ATSDR, 2004).Although copper homeostasis plays an important role in the prevention of copper toxicity, exposure to excessive levels of copper can result in a number of adverse health effects including liver and kidney damage, anaemia, immunotoxicity and developmental toxicity. Many of these effects are consistent with oxidative damage to membranes or macromolecules. Copper can bind to the sulfhydryl groups of several enzymes, such as glucose-6-phosphatase and glutathione reductase, thus interfering with their protection of cells from free radical damage (ATSDR, 2004). A few examples of human activities that contribute to copper release are mining, metal production, wood production and phosphate fertilizer production. Because copper is released both naturally and through human activity, it is very widespread in the environment. The metal is often found near mines, industrial settings, landfills and waste disposals (Schult and Kelling, 1999)

1.3.4 ZINC (Zn)

Zinc was essentially established in 1869 for plants, in 1934 for experimental animals and in 1961 for human. Since the discovery of zinc deficiency as a human problem in 1961, interested in the biochemical and clinical aspects of zinc nutrition has increased markedly. In this paper, zinc is review in biochemical and physiological functions, metabolism including, absorption, excretion homeostasis, zinc bio availability (inhibitors and enhancers), human requirement, group at high-risk consequences and causes of zinc deficiency, evaluation of zinc status, and prevention strategies of zinc deficiency (Nazanin et al., 2013).

1.3.5 LEAD (Pb)

Lead is a soft heavy metallic element, it was one of the nine chemical elements known and used in the ancient world throughout history it has been widely employed in arts and architecture and in modern time, it has been used in making shield reaction. It is used as element in lead storage batteries, solder, plumbing, cable covering is also recognized. It is used in interior paints; it causes serious hazard because lead highly toxic if improperly handled (wajahat et’al 2006) lead in the solid is found into solution either by digesting it with hydrochloric acid solution. The extract is used to determine the element using atomic absorption spectroscopy (AAS) the element has a wavelength of 280mm (wajahat et’al 2006).

Lead can cause several health problems at relatively low level of exposure. It is found in organic form, inorganic leads affect typically the nervous system (CNS), peripheral nervous system (PNS), haematopoietic, renal, cardiovascular and reproductive system. Organic leads toxicity tends to predominantly affect the central nervous system. Inorganic lead enters the body by way of ingestion or inhalation, for adult only about 10% of the ingestion dose is absorbed in contrast, children may absorb as much as 50% of ingested dose. lead affect the production of blood cell, kidneys and behaviour it poses the placenta and can damage the foetal nervous system this disease of the nervous system can affect every aspect of reproduction in woman from fertility to lactation, and maternal neurologic disease and its treatment may have an impact on the offspring of that pregnancy. (Michael et al., 2008).

Lead (Pb) is a well-known neurotoxin. Impairment of neuro development in children is the most critical effect of lead poisoning. Exposure in the uterus, during breastfeeding and in early childhood may all be responsible for the effect. Lead accumulates in the skeleton and its transfer from bones during pregnancy and lactation causes exposure to foetuses and breastfed infants (ATSDR, 2007). Chronic exposure to Pb can affect physical growth and cause anaemia, kidney damage, headache, hearing problems, speaking problems, fatigue or irritable mood (Simeonov et al., 2010). The toxicity by Pb has multiple biochemical effects. It has the ability to inactivate enzymes, compete with calcium for incorporation into bones and interfere with nerve transmission and brain development (Ediin et al., 2000). It has been suggested that lead on a cellular and molecular level may permit or enhance carcinogenic events involved in DNA damage, DNA repair and regulation of tumour suppressor and promoter genes (Silbergeld, 2003).The common sources of lead are car batteries, tyre materials, coals, plastics and insecticides. The high level of Pb in soil, could be attributed to Pb from car exhaust fumes not lined nor basement prepared for selective adsorption of toxic substances. Therefore, it is prone to release pollutants to nearby water and to the air through leachates and dumpsite gases respectively (Alimba et al., 2006). Many water resources have been rendered wholesome and hazardous to man and other living systems, as a result of indiscriminate dumping of refuse (Bakare et al., 2005).Most uncontrolled dumps are many years old, having grown over time from small dumps to large, unmanaged waste sites. Uncontrolled dumps have significant environmental impacts. Solid waste poses the greatest threat to life since, it has the potential of polluting the terrestrial, aquatic and aerial environments (Odukoya et al., 2000). Soil samples represent an excellent media to monitor heavy metal pollution resulting from anthropogenic activities. Heavy metal contaminated soil affects the ecosystem due to leaching into ground water or when they are taken up by plants and animals, which results in great risks due to bioaccumulation (Bhagure and Mirgane, 2010). Heavy metals and persistent organic pollution are of concern due to their potential harmful effects to humans and the environment. Wong et al. (2003) showed that heavy metals are potentially toxic to crops, animals and humans when contaminated soils are used for crop production. Pollution of the biosphere with heavy metals, induced by industrial, agricultural and domestic activities pose serious problems for safe use of agricultural lands (Fytianos etal., 2001). In some dumpsites, wastes are burnt in the open and the ashes abandoned at the sites, with no regard to environmental implications. This poses a direct safety threat because of the danger of explosion. The burning of wastes gets rid of the organic materials and oxidizes the metals, leaving the ash richer in metal contents. After the processes of (Momani, 1999). Cd is also found in lubricating oils as part of many additives and car tyres as a result of the vulcanization process. The industrial activities; the metal is widely used in electroplating, pigments, plastics, stabilizers and battery industries (Mehbrahtu and Zebrabruk, 2011). Cd is highly toxic and responsible for several cases of poisoning through food. Small quantities of Cd cause adverse changes in the arteries of human kidney. It replaces zinc biochemically and causes high blood pressures and kidney damage (Mehbrahtu and Zebrabruk, 2011). The recommended concentration of cadmium in soil is 3kg/mg (EU, 1986).1.3.6 IRON (Fe)

Iron is a chemical element with symbol Fe (from Latin: ferrum) and atomic number 26. It is a metal that belongs to the first transition series and group 8 of the periodic table. It is, by mass, the most common element on Earth, right in front of oxygen (32.1% and 30.1%, respectively), forming much of Earth's outer and inner core. It is the fourth most common element in the Earth's crust.

In its metallic state, iron is rare in the Earth's crust, limited mainly to deposition by meteorites. Iron ores, by contrast, are among the most abundant in the Earth's crust, although extracting usable metal from them requires kilns or furnaces capable of reaching 1,500 °C (2,730 °F) or higher, about 500 °C (900 °F) higher than that required to smelt copper. Humans started to master that process in Eurasia by about 2000 BCE,[not verified in body] and the use of iron tools and weapons began to displace copper alloys, in some regions, only around 1200 BCE. That event is considered the transition from the Bronze Age to the Iron Age. In the modern world, iron alloys, such as steel, stainless steel, cast iron and special steels are by far the most common industrial metals, because of their mechanical properties and low cost.

Pristine and smooth pure iron surfaces are mirror-like silvery-gray. However, iron reacts readily with oxygen and water to give brown to black hydrated iron oxides, commonly known as rust. Unlike the oxides of some other metals, that form passivating layers, rust occupies more volume than the metal and thus flakes off, exposing fresh surfaces for corrosion. Although iron readily reacts, high purity iron, called electrolytic iron, has better corrosion resistance.

The body of an adult human contains about 4 grams (0.005% body weight) of iron, mostly in hemoglobin and myoglobin. These two proteins play essential roles in vertebrate metabolism, respectively oxygen transport by blood and oxygen storage in muscles. To maintain the necessary levels, human iron metabolism requires a minimum of iron in the diet. Iron is also the metal at the active site of many important redox enzymes dealing with cellular respiration and oxidation and reduction in plants and animals.

Chemically, the most common oxidation states of iron are iron(II) and iron(III). Iron shares many properties of other transition metals, including the other group 8 elements, ruthenium and osmium. Iron forms compounds in a wide range of oxidation states, −2 to +7. Iron also forms many coordination compounds; some of them, such as ferrocene, ferrioxalate, and Prussian blue, have substantial industrial, medical, or research applications.

1.3.7  NICKEL (Ni)Nickel is an essential trace element in animals, although the functional importance of nickel has not been clearly demonstrated. However, there is evidence of uptake and accumulation in certain plants. Nickel deficiency is manifested primarily in the liver. Its effects include abnormal cellular morphology, oxidative metabolism and increase and decrease in lipid levels. The essentiality of nickel in humans has not been established and nickel dietary recommendations have not been established for humans (ATSDR, 2003).Nickel compounds are known carcinogens in both human and animal models (Feder et al., 1996). There is evidence that the geno-toxic effects of nickel compounds may be indirect through the inhibition of DNA repair systems. As a result of this inhibition it has been suggested that accumulation of nickel in breast tissue may be closely related to malignant growth process (ATSDR, 2005).1.3.8 ARSENIC

Arsenic occur naturally in the environment on the earth crust, it is an essential element that becomes toxic when ingested at high level, Arsenic is combined with other elements such as oxygen, chlorine, sulphur to form inorganic arsenic compounds. Arsenic enter into the environment primarily from air and by seeping in to the soil and ground water from hazardous water dumps (kristeen cherney2018).

Arsenic poisoning, or arsenicosis, occurs after the ingestion or inhalation of high levels of arsenic. Arsenic is a type carcinogenic that’s gray, silver, or white in colour. Arsenic is extremely poisonous to humans. What makes arsenic especially dangerous is that it doesn’t have teste or odour, so you can be exposed to it without knowing it. (kristeen cherney2018). Some toxic effects of arsenic poisoning are gastrointestinal complain, diarrhoea, constipation, gastrointestinal, disturbance, loss of appetite, and weight loss exposure to arsenic at low level for extended period of time can cause discordant for skin and the appearance of small corns and warts. Exposure to high level of arsenic can lead to death (Ebong et al., 2008).

1.4 AIMS AND OBJECTIVES OF THE STUDY

The aim of this research is to determined the level of some heavy metals in some selected dump site soil samples in bajoga town of gombe state, Nigerian with following sets of objectives.

(i) To collect the soil samples in the v.cinity of the selected dump sites in bajoga town.

(ii) To determine the level of concentration of the heavy metals ( Cd,Cr,Cu,Pd,Ar,Ni and Fe) in the various dump sites.

(iii) To proffer recommendations based on the findings of the research.

1.5 STATEMENT OF THE PROBLEMS

Like many cities in Northern Nigeria, Bajoga town in Gombe  State faces problems of environmental sanitation such as improper disposal of refuse near residential areas; poor refuse collection and handling, it is common to find huge refuse dumpsite within

residential areas and farmers use them as fertilizers. This however, leads to accumulation of heavy metals in plants grown on dumpsite soils or fertilsed with dumpsite manure, which on subsequent transfer through food chain end up in man, posing potential health risk.

The rise in population and civilization in Bajoga town has increased the number of dumpsite due to poor waste management schemes. It is a common practice to burn dumpsite wastes, this burning gets rid of organic matter and become ashes which are richer in metal contents. These ashes are either dissolved in rain water and leached into the soil contaminating the underground water, or washed away by runoff into streams and rivers, thereby contaminating the environment, it is based on these facts that this study is aimed at determining the total concentration of cadmium, copper, nickel, lead and chromium in dumpsite soils and Abelmochus esculentus fruit planted on dumpsite soil in Bajoga town, Gombe  State, Nigeria.

1.6 SIGNIFICANCE OF THE STUDY

The research will help the people of Bajoga to be aware with the effects and maintenance of heavy metals in human body and environmental conditions.

1.7 SCOPE AND LIMITATION OF THE STUDY

This project limits its work on six (6) heavy metals in some selected soil of the study area of Bajoga town of Funakaye Local Government Area namely:

- Unguwar Sarkin Yaki

- Unguwar Zamfara

- Unguwar Isah

- Unguwar Barde-kolo

- Unguwar Leggel

- Unguwar Bamalu

CHAPTER TWO

2.0 LITERATURE REVIEW2.1 MUNICIPAL SOLID WASTE MANAGEMENT

Municipal solid waste management is a major responsibility of state and local government agencies. In order to curtail the problems associated with solid waste generation and other related problems, the Federal Government of Nigeria has promulgated various laws and regulations to safeguard the environment. The Federal Environmental Protection Agency Act of 1988, pursuant to the FEPA Act, each state and local Government in the country set up its own environmental protection body within its jurisdiction. The agencies arecharged with the responsibility of handling, employing and disposing of solid waste generated. The state and local government agencies generate fund from subvention from internally generated revenue through sanitary levy and stringent regulations with heavy penalties for offenders of illegal dumping and littering of refuse along streets (Ogwueleka, 2003). Waste handling facilities are lacking in many highly populated areas in most developing and underdeveloped countries due to cost and lack of enforcement of relevant enactment. Poor regional and urban planning, lack of enforcement of relevant laws and edicts on waste disposal, lack of organized landfill sites contribute to the presence of dumpsites within living areas in developing nations. This results in the discharge of household sewage and refuse into the environment untreated. The surface runoff and leachates from dumpsites are sources of fresh water contamination (Abdus-Salam et al., 2011).The recent population and industrial growth has led to increasing production of domestic, municipal and industrial wastes, which are indiscriminately dumped in landfill and water bodies without treatment (Ogunyemi et al., 2003). Municipal solid waste management constitutes one of the most crucial health and environmental problems facing governments of African cities. This is because even though these cities are using 20-50 percent of their budget on solid waste management, only 20-80 percent of the waste is collected. The uncollected or illegally dumped wastes constitute a disaster for human health and result to environmental degradation (Achankeng, 2003). Thousands of old landfills and dumpsites exist throughout developing countries that may pose threat to human health in the next decades, unless appropriate measures are taken. Most developing countries follow the practice of open dumping of solid wastes causing environmental and health risks(Kurian et al., 2003). Industrialization, population growth and unplanned urbanization have partially or totally turned our environment to dumping sites for waste materials (Ikem et al., 2002). A metal of relative high density (specific gravity greater than about 5) or of high relative atomic weight is defined as heavy metal. The term heavy metal is used to describe more than a dozen element that are metals or metalloid (vogels 1989) e.g chromium, arsenic, cadmium, lead, zinc   nickel e.t.c heavy metals are natural constituents of the earth crust because they cannot be degraded or destroyed heavy metals are persisted in all part of the environment. In small amounts they enter the human body via food, drinking water and air. living organism require varying amounts of heavy metals iron, cobalt, copper, manganese, molybdenum and zinc are required by human. excessive levels can be described as many metallic elements that has a relatively high density and is toxic or poisonous at low concentration.

Human activities effect the natural geological and biological distribution of heavy metals through pollution of air, water, and soil. Humans are also responsible for altering the chemical formed of heavy metals released to the environment. Such as alterations of often effects a heavy metals toxicity by hollowing it to bioaccumulation in plants and animals. Bio concentrates in the food chain or attack specific organ of the body bioaccumulation refers to an increase in the concentration of heavy metal in biological organism or targeted organ over time until they become hazardous to health (mata et al.,2008). Many metals and other chemicals accumulates in living things anytime they taken up and store faster than they are broken down (metabolized or excreted).

2.2 BEHAVIOUR OF INORGANIC (HEAVY METAL)CONTAMINATION IN THE SOIL

The most prevalent group of elements in the sub soil is the transition metals otherwise called heavy metals. Example includes zinc (Zn), arsenic (As), lead (Pb), nickel(Ni), cadmium(Cd), chromium(Cr) etc. these metals may occur in soil due to industrial or from refused dumps.

2.3 EFFECTS OF MUNICIPAL SOLID WASTE ON THE ENVEROMENT

Poor waste management poses a great challenge to the well-being of city residents, particularly those living adjacent to dumpsites due to the potential of the waste to pollute water, food, land, air and vegetation. The poor disposal and handling of waste thus leads to environmental degradation, destruction of the ecosystem and may cause great risks to public health. The resultant accumulation of waste poses a health hazard to urban inhabitants, and also threatens the surrounding environment (UNDP, 2006).The environmental problems posed by solid waste ranges from health hazards, soil and water pollution, repulsive sight and offensive odour. The resultant effect of these is the degradation of environmental quality (Abdus-Salam et al., 2011). Dumpsites are usually located within the vicinity of living communities and wetlands. These dumpsites are often not lined nor basement prepared for selective adsorption of toxic substances. Therefore, it is prone to release pollutants to nearby water and to the air through leachates and dumpsite gases respectively (Alimba et al., 2006). Many water resources have been rendered wholesome and hazardous to man and other living systems, as a result of indiscriminate dumping of refuse (Bakare et al., 2005).

2.4 SOIL POLLUTIONSoil is a very important natural resource to man as it is a source of life on planet, earth. Without soil, the earth would be as barren as the moon, hence lifeless (Misra and Mani, 2009). Despite its importance, soil is often contaminated by human activities and this is reflected in the high horizontal and vertical variability brought about by the anthropogenic influence on soil formation and development (Fong et al., 2008). A variety of human activities including municipal waste disposal, industrial emissions, military testing and agricultural practices have left their impacts on soils in the form of elevated and high level of toxicants (Van and Krivolutsky, 1996). Materials that find their entry into the soil system persist and accumulate in toxic levels, hence becoming sources of pollution in the soil (Misra and Mani, 2009). The concentration of heavy metals in soil and their impact on ecosystems can be influenced by many factors such as the parent rock, climate and anthropogenic activities (Jia et al., 2010).

2.5 EXPOSURE OF HUMAN TO HEAVY METALS

Since the industrial revolution, the production of heavy metals such as lead, zinc etc. has increased exponentially, production of this metals has increase nearly 10 – fold with emissions rising in tandem, once emitted, metals can reside in the environment for hundreds of years or more human beings may be exposed to these metals through the food chain after the food has been contaminated. Heavy metals have been used in a variety of ways for at least two millennia, for example, lead has been used in plumbing and lead arsenate has been used to control insects in apple orchards. Lead was added to wine to improve its taste and mercury was used to alleviate teething pair in infants, many metals are essential to life and only become toxic when exposures to life become excessive. (i.e. exceed some threshold for the introduction of adverse effect) (Nwajei et al., 2007).

Exposure to heavy metals has been linked with developmental retardation, various cancers, kidney damage and even death in some instances of exposure to high concentrations. Despite abundant evidence of these deleterious health effects, exposure to heavy metals continues and may increase in the absence of concerted policy actions. Exposures to unhealthy elements remain common throughout both developed and developing countries. Unlike most chemicals for which health impacts of low level dose are still uncertain, exposure of lead even at low level is highly toxic

(Ademoroti, 1996). The natures of biological response to metal exposures are direct consequences of exposure and are defined through dose-effect relationship.

2.6 BIOACCUMULATION OF HEAVY METALS IN LIVING SYSTEM

The word environment and society where we live have made bodily contamination impossible to ignore – through eating, smoking, breathing, skin absorption and every day exposure to limitless products and chemicals made and used by human, contaminants find their way into the body. Over time  these heavy metals, toxic chemicals and residues, plagues and other natural intruder continue to slowly accumulate. If the body natural detoxification pathways (such as blood lymph and cerebral spinal fluid) cannot eliminate them faster that they enter the body, the build up can eventually reach toxic levels.

“Bioaccumulation of metals is the process whereby organisms stored up metals from the surrounding medium into their tissues by chelating process”.  It is the product of equilibrium between the concentration of the metal in organisms, environment and the rate of ingestion and excretion, for example inorganic mercury present in product like pesticide, insecticides, fungicides etc. usually deposit in water to form sediments. These chemicals may be absorbed by Aquatic organisms and the organisms are eventually eaten up by fish, the fish are caught and eaten by man, as such the chemicals may bio-accumulate in man on exposure for a long time. The consequence of this may result to various ailments which may sometime life threatening (Osuj & Onojake, 2004).

2.7  NATURAL CHELATING AGENTS

The word, chelates, means to grab onto something. Thus chelating agents are substances that have a strong ability to grab onto metals and dislodge them from the tissues so they can be removed. The human body has its own cleansing mechanism utilizing chelating agents. Each cell in the body manufactures its own chelating agents such as cysteine, histidine, glutathione  and other metallothioneins designed to remove heavy metals and other toxins. These chelating agents involved in natural chelation are amino acids. Our bodies can only synthesize these amino acids from a sufficient amount of dietary protein. For example, our bodies synthesize cysteine from the amino acid methionine found in garlic and onions. Additionally, sugars, lipids (i.e. fat) and other proteins can act as chelating agents. As we age, our cells produce less and less of these chelating agents. So, over our lifetime, supplementation becomes increasingly important to remove heavy metals and other toxins from our bodies. For instance, older adults should greatly increase their supplementation of these chelating agents from plants, clay and fermented foods. Therefore an increased dietary intake of plant and animal proteins is highly recommended to keep natural chelation going otherwise the body may be exposure to toxins ingested.

2.8  REVIEW OF RELATED WORKS

Umoh & Etim (2013) reports that the concentration of metals in soils at the decomposed biodegradable waste dumpsite and 100m away (control site) from dumpsite indicate that there is a relative increase in the concentration of  heavy metal at dumpsites compared to those in soils 100m away (control site) from the dumpsite. This is in agreement with the results obtained from similar a study by Amusan et al., (2005). This could be attributed to the availability of  metals containing wastes at dumpsite which are eventually leached into the underlying soils. The metals considered in this study include the metals which are micro-nutrient such as iron, zinc and copper and the non-essential/toxic heavy metal which are toxic to plant when present in the soil at  concentrations above tolerance level. These elements are highly needed by plants for their normal growth. From the results obtained, the concentration of  lead at dumpsites ranged from 9.46 mg/kg to 18.83mg/kg and for – the control sites it ranged from 5.21 mg/kg to 7.53mg/kg showing that both the dumpsites and control sites had normal range of 2-200mg/kg (Ebong et al., 2008). Comparing values at dumpsites with that at control sites, the higher concentration of lead at dumpsites could be attributed to the decomposition of lead containing wastes. From similar work reported by Dara (1993), the major source of lead pollution is industrial source. Thus, any nearby industry that  disposes lead as one of its wastes can also influence the concentration of lead in that environment. According to Aluko et al.,(2003) the mean concentration of lead in soil at Ibadan dumpsite also ranged from 1.34mg/g to1.693mg/g. But since lead is a cumulative pollutant (Dara, 1993) the pollution of soil by lead remains a very serious problems that should be given much attention by environmental chemists in collaboration with government agencies. Also, effort should be made to educate the public on the health effect of this metal when ingested in excess. Such effects which includes: damage of the brain, kidney, miscarriage in pregnant women and damage of sperm production organ in male (Sabine and Wendy, 2009). The standard and accepted level of iron in soil ranges between 100-700mg/kg (Ebong et al., 2008). From this study, the dumpsite at Udua Adan Market recorded the highest level of iron (23.47mg/kg) while the dumpsites at Ukam Street recorded the least level of iron (18.06mg/kg). The five dumpsites analysed for iron ranged between (18.06 to 23.47) mg/kg which fell far below the accepted range of iron in soil. At the control site, the concentration also ranged between (8.24 to 11.72) mg/kg, which is much far below the accepted range of iron in soil. But levels iron recorded was higher than the levels of all the metals analysed. This indicates the importance of iron to humans.

According to WHO (1984), the deficiency of iron in man can cause weak muscular coordination, vomiting, diarrhoea and other serious health defects.  Comparing this with the result obtained by Akaeze (2001), Elelenwo (of Rivers State) dumpsite had concentration of iron range between (10,300 -31,000) ppm which falls within accepted level. But from studies carried out by Udeme (2001) for soil along Abak/Ikot Ekpene Road, in Uyo metropolis, Akwa-Ibom State Nigeria, using different methods at different sample location revealed results that are comparable to the one obtained in this study. Eddy et al., (2004) suggested that different sample location revealed results that are comparable to the one obtained in this study. Eddy et al., (2004) suggested that any pollution of the environment by iron cannot be conclusively linked to waste materials alone but other natural sources of iron must be taken into consideration. From the authors’ point of view: despite the fact that iron is a micro nutrient, it should be properly monitored to maintain its concentration in the accepted range to avoid health defect caused by the deficiency or excess amount of it.

Cadmium is classified as a soft acid, preferentially complexing with sulphides (Moore, 1991). Its accepted range in soil as stated by Ebong et al., (2008) is (0.01 - 300) mg/kg. The result obtained showed that the cadmium level at dumpsite and control site, both fall below accepted range of the metal in soil. From the analytical result, it was found that the range of cadmium at dumpsite within the study area falls between (0.09 to 0.42) mg/kg and that of its control site falls between (0.04 to 0.08) mg/kg. Cadmium was listed by EPA (1991) as one of the 129 priority pollutants and among the 25 hazardous substances. Ingestion of high level of cadmium severely irritates the stomach leading to vomiting and diarrhoea. Cadmium and its compounds are known human carcinogens and smokers get exposed to significant amount of  cadmium than non-smokers. Other effects associated with cadmium include damage of lungs, fragile bones and kidney disease (Sabine and Wendy, 2009). Thus, should be able to attract the attention of environmental chemists, Government agencies and other private bodies.

The concentration of zinc in soils was obtained to range between(13.82 to 17.26) mg/kg for dumpsites and (6.32 to 8.15) mg/kg for the control sites. It could be seen that this falls within the accepted standard for the dumpsites but slightly deviate for the control sites. Control sites for kent Street, Udua Adan market and Ukam Street were specifically found to fall out of the accepted range of zincs in soil which is (10 - 300) mg/kg. According to Odukoya et al., (2000), the range of zinc obtained from Abeokuta dumpsites was 100.80 to 226.00 mg/kg, while its control sample range was found to be (51.25 to 71.43) mg/kg which falls within the accepted range. This variation could be attributed to the fact that categories of waste introduce to such

dumpsites have high concentration of zinc. Also from the concentration of  zinc in the two control soil samples can be high due to some environmental factors. This was confirmed by Dara (1993). Zinc is required in human nutrient for normal functioning of the body. The deficiency of zinc in man can lead to impaired growth, low energy balance and low protein intake. While excessive intake of zinc from plants can lead to vomiting, dehydration, electrolyte imbalance, abdominal pain, and lack of muscular co-ordination (Udosen, 2000). The natural range of concentration of copper in soil is (2-100) mg/kg (Ebong et al., 2008) the concentration range between (6.68 to11.4)mg/kg and (2.06 to 5.61) mg/kg for dumpsite and control site respectively of the study area. Both the dumpsites and control sites concentration of copper within the study area falls within the accepted standard range. There is no deleterious effect of copper resulting from its deficiency or excess amount. According to Dara (1993) the high concentration of copper at the dumpsite might be attributed to biodegradable

waste introducing metallic copper into the soil. World Health Organization (1984) stated that, the injection of copper can lead to severe muscular irritation, nausea, vomiting, diarrhea, intestinal cramps, severe gastrointestinal irritation, and other dangerous health defects. For the general results obtained from this study, the top layer (0-15cm depth) of the soil was employed in the research since earlier studies by Nyangababo and Hamya (1986) indicated the top soil layers as better indicators of metallic burdens. From the study (Umoh & Etim, 2013), he observed that since none of the heavy metals analysed was higher than the accepted standard, there was no hazard associated with the high concentration of the metals analyzed but the concentrations of all the heavy metals analysed at dumpsites were higher than the values at the control sites which might not be unconnected with the rate or level of leaching in the soil. An evidence of this relative increase in concentrations of metals in soils at dumpsites compared to those 100m away from dumpsites shows that the concentration of heavy metal in soil at  dumpsite is directly proportional to its concentration distance away from dumpsite provided porosity is constant. Thus dumpsites have a significant impact in the environment which could be positive or negative. This means that there should be a kind of measure refuse to prevent the general public from being exposed to unnecessary hazards. Hence, residential areas should be sited a reasonable distance away from dumpsite especially when the need for drilling bore hole for town water supply arises, environmental impact assessment is highly recommended. Effort should also be made to discourage the practice of  cultivation at dumpsite soil (Ebong et al., 2008) since plants growing at  dumpsite soil bio accumulate considerably high heavy metal content than those in normal soil and the concentration of heavy metal on this soil can easily increase beyond tolerance level causing serious problems along the food chain. Thus the need to test for the physicochemical parameters of  waste and the concentration of heavy metals in waste before dumping in a different study remains an outstanding area which should be focused on (Umoh & Etim, 2013).

Ideriah et al., (2005) studied the levels of some heavy metals in soils around designated municipal solid waste dumpsite and a control site within Port  Harcourt and its environs. The soil samples were randomly collected and analyzed for As, Cd, Cu, Cr, Ni and Pb. Levels from the waste dumpsite were higher than those from the control site. Soils around the waste dump were also contaminated as a result of continuous dispersion of heavy metals from the waste dump by run-off water, wind and scavengers. The difference in the mean concentrations of metals between the main dump and outside the dump were not significant (P>0.05) in the wet season but were found to be significant (P<0.05) in the dry season. The concentrations of the metals in soils in both seasons ranged from 0.50 ȝg/g to 20.5 ȝg/g for As, 0.20 ȝg/g to

13.0 ȝg/g for Cd, 0.50 ȝg/g to 100.0 ȝg/g for Cr, 2.50 ȝg/g to 910.0 ȝg/g for Cu, 0.50 ȝg/g to 34.0 ȝg/g for Ni and 1.0 ȝg/g to 127.5 ȝg/g for Pb. The difference observed in the concentrations of Cd, Cu and Pb in both seasons are statistically not significant (p > 0.05 Pb) while the seasonal variations in the concentrations of As, Cr, and Ni show signification difference (P<0.05).

Cd, Cr, and Cu showed positive correlation with distances across the waste dump (r = 0.9236, 0.9338, 0.3586 respectively). The mean concentrations of  Cd and Cu in the soils from the waste dumpsite are sufficiently high to cause environmental concern as their concentrations exceeded tolerable limits (Ideriah et al., 2005).



CHAPTER THREE

3.0 METHODOLOGY

This chapter focuses on the study area, sampling technique, sample collection, apparatus/reagent, sample preparation, method of sample analysis and data analysis.

3.1 STUDY AREA

Bajoga the headquarter of Funakaye local government area of Gombe state lies on the geographical coordinate of latitude10’51’15N and longitude 11’25’54E and it covers a long area of 3.408km2 with a population of 166,630, its located in the northern part of Gombe, Nigeria, its 9km south of the Ashaka cement factory, Funakaye local government area, its bounded in the East by Gongola river, and a lake Dadinkowa beyond which lies between Yobe state and Borno state.

3.2 SAMPLE TECHNIQUE

Bajoga town was demarcated into six (6) major areas for this study.

The areas include Unguwar Sarkin Yaki, Unguwar Zamfara, Unguwar Isah, Unguwar Barde-kolo, Unguwar Leggel, Unguwar Bamalum respectively. The samples were taken from these four (4) locations to identify the following heavy metals Cd, Cr, Pb, Zn, As, Ni.

3.3 SAMPLE COLLECTION

The soil samples were collected around a particular refuse dump side, plain clear field and were blended (mixed) to obtain a representative sample.  The samples were obtained at the depth of 10-12cm by the use of machete for digging; trowel for fetching and the meter rule for measurement. The sample collected, were package in polyethene bags /bottles and then taken to the laboratory for digestion and analysis.

3.4 APPARATUS AND REGENT

Sampling is carried out with a number of tools, depending upon the purpose for which sampling is done and the nature of the sample

Equipment/apparatus used for the collection of samples in this research work as well as theses used for preparation of the sample include:

- Machete

- Meter rule

- Polyethene bags/bottles

- Hand Trowel

- 100ml volumetric flask

- 250ml beakers

- 100ml measuring cylinder

- Filter paper

- Fume cupboard

- Hot plate

- Mesh sieve(2mm)

- Distilled water

- Concentrated HCL

- Concentrated HNO3

3.5 SAMPLE PREPARATION

Sample were mixed frequently to expose fresh surface to dryness. The sample were mixed, grinded after drying and sieved through a 2mm mesh sieve and the coarse particle discarded. The powdered form was then stored for digestion and subsequent analysis.

3.6 SAMPLE DIGESTION

METHOD

The conventional aqua regia (i.e solution of HCL and HNO3 with the ratio of 3:1) digestion was employed. A well-mixed sample of 2g was digested in 24ml of aqua regia on a hot plate for 5minutes. After evaporation to near dryness, the sample was diluted with 20ml of distilled water and transferred into a 100ml volumetric flask after filtering through whatman No 1 filter paper. The extract were then taken for subsequent analysis of heavy metals [chen,2001]

3.7 METHOD OF SAMPLE ANALYSIS

Atomic absorption spectrophotometer (AAS), was used for the sample analysis.

3.7.1 PRINCIPLE

Atomic absorption spectroscopy is based on the ability of an ‘excited’ atom of an element to absorb energy from wavelength of light of the same frequency as the element. This creates a decrease in the initial signal energy and this difference is proportional to the concentration of the element

Each element has its own series of specific characteristic of sensitivity; noise and linearity define the range in which calibration curve will be accurate.

The optical system is set up with a hollow cathode lamp for sample of interest (analysis). With the appropriate slit and wavelength selected for the element, a solution of known concentration of the analysis (standard) is aspirate and the standard and the absorbance reading is rational to the standard and the sample concentration is interpolated from the intensity of the reading.

Atomic absorption spectrophotometer (AAS) has various models, but for this analysis, buck model 210VGP system was used. The model has the advantage over other models because it operates in three (3) modes; direct measurement, deuterium background correction(dz) and variables giant pules correction (gp). The two correction modes allow the chemist to analyse sample that has complex or ‘’dirty’’ matrices; which would normally give significant interference in the analytical determination (Ajayi and kamson , 1983).

CHAPTER FOUR

4.0 RESULT AND DISCUSSION

This chapter present and discuss results of the findings of this work

4.1 TABLE OF RESULT

Table 1. the concentration and mean concentration of heavy metals in the six (6) locations

| Sample Site | HEAVY METALS CONCENTRATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Fe Mean + SD | Cd Mean + SD | Cr Mean + SD | Ni Mean + SD | As Mean + SD | Pb Mean + SD |
| S1 | 0.0811 0.0003 | 0.224 0.0004 | 0.0191 0.0007 | 1.124 0.0113 | 0.0305 0.0004 | 0.0733 0.0003 |
| S2 | 0.1051 0.0012 | 0.0351 0.0001 | 0.2141 0.0006 | 3.616 0.0488 | 0.0420 0.0010 | 0.0253 0.0003 |
| S3 | 0.1648 0.0031 | 0.0210 0.0002 | 0.1455 0.0039 | 2.077 0.0221 | 0.0287 0.0002 | 0.0133 0.0006 |
| S4 | 0.1030 0.0011 | 0.0375 0.0007 | 0.2055 0.0006 | 1.836 0.0148 | 0.0225 0.0006 | 0.0164 0.0006 |
| S5 | 0.0961 0.0007 | 0.0289 0.0004 | 0.3142 0.0025 | 2.119 0.0085 | 0.0205 0.0000 | 0.0144 0.0002 |
| S6 | 0.1030 0.0028 | 0.0215 0.0007 | 0.0324 0.0004 | 1.456 0.0368 | 0.0176 0.0005 | 0.0182 0.0005 |
| WHO STANDARD | 425.5μg/g | 0.02-0.5 | 0.002-0. | 0.1-5 | 0.009-1.5 | 0.3-10 |

Key

S1=UNGUWAR ZAMFARA

S2=UNGUWAR SARKIN YAKI

S3=UNGUWAR LAGGEL

S4=UNGUWAR ISAH

S5=UNGUWAR BARDE KOLO

S6=UNGUWAR BAMALUM

4.2 DISCUSSION

The concentrations of heavy metals in dumpsites within the study area were determined using Atomic Absorption spectrophotometer (AAS). The study revealed that the concentrations of metals in these sites investigated;

Fe,Cd,Cr,Ni,As,Pb, were above the established threshold limits set by WHO.Samples from S1 show the concentration (mg/kg) of heavy metals as follows: Fe (0.0003 mg/kg) Cd (0.0004 mg/kg), Cr (0.0007 mg/kg), Ni (00113 mg/kg), As (0.0004 mg/kg), Pb (0.0003 mg/kg); while the samples from S2 indicate the concentrations of Fe (0.0012 mg/kg), Cd (0.0001 mg/kg) Cr (0.0006 mg/kg), Ni (0.0488 mg/kg), As (0.0010 mg/kg) Pb (0.0003 mg/kg). While Samples from S3 indicated the concentration (mg/kg) as shown: Fe (0.0031 mg/kg), Cd (0.0002 mg/kg), Cr (0.0039 mg/kg), Ni (0.0221 mg/kg), As (0.0002 mg/kg), Pb (0.039 mg/kg).Further more soil samples from S6 shows the concentrations (mg/kg) Fe (0.0028 mg/kg) Cd (0.0007 mg/kg) Cr (0.0004 mg/kg) Ni (0.0368 mg/kg) As ( 0.0005 mg/kg) pb (0.0005 mg/kg).The results from S4 indicates the concentrations(mg/kg) of heavy metals as Fe (0.0011 mg/kg), Cd (0.0007 mg/kg), Cr (0.0006 mg/kg), Ni (0.0148 mg/kg), As (0.0006 mg/kg), Pb (0.0006 mg/kg). While the samples from S5 indicate the concentrations (mg/kg) : Fe (0.0007 mg/kg), Cd (0.0004 mg/kg), Cr (0.0025 mg/kg), Ni (0.0085 mg/kg), As (0.0000mg/kg) and Pb (0.0002 mg/kg). All the heavy metals was detected in all the dumpsites; Pb was detected in all the dumpsites; it appears to be lower concentration in all dump site . This can be attributed to the disavailability of lead containing waste at the dumpsites which are eventually leached into the underlying soils. It could also be as a result of their proximity to the highways or heavier dumpsites that carries heavier traffic (Bradford et al., 1975). Lead level of 10ȝg/dL or above is a cause for concern. Lead hadbeen is known to have harmful health effects even at lower levels and there is no known safe exposure level. It is pertinent to note that exposure to amount of lead above 0.01mg/kg is detrimental to health, as it may result in possible neurological damage to foetuses, abortion and other complication in children under three years old (Asemave et al., 2012). Nickel (Ni) was found in higher concentration in dumpsites of S1 follow by S6.This implies that waste carrying Ni are not present in it much quantity. The low concentration could be accorded to the absence of industries and smelting activities in S5 area. Chromium (Cr) was also detected in all the dumpsites, though generally low and this can also be attributed to the dumpsite carrying waste of low Cr concentration. This may have been less human activities capable of generating chromium. Chromium +3 is less damaging to the health due to their absorption by the body (<1%), but Cr +6 is acutely poisonous and on contact with the skin, it triggers dermatitis, allergies and irritations, thus considered as carcinogenic to humans (Asemave et al., 2012).Cadmium (Cd) were also detected in all the dumpsites as their concentrations were low. This can also be as a result of low Cd carrying waste. This is low compare to WHO standard of:Cd = 0.02 – 8.2. mg/kg. Uba et al., (2008) reports that, Cd has highest level bioavailability in dumpsites. In general, the Concentrations of Cd may be attributed to various sources, some of which include automobile tire dust, burning of oil and tyre, plastic wrappings, paints, dyes, and especially, refuse dumps and commercial activities (Ali et al., 2005). The levels of these heavy metals detected in these refuse dumpsites fall within the allowable limit set by the world health organization (USEPA, 1986). Arsenic (As) was detected in all dumpsite, it was found to be relatively low as compare to WHO standard (As = 0.009 – 1.5 mg/kg) for heavy metal concentration in the soil. The results of heavy metals obtained from the analysis of soil around refuse dump in S1,to S6, indicate that their concentrations are relatively low. The low concentration could be as a result of waste carrying less concentrations of these metals (Fe, Pb, Ni, Cd, Cr and As) in all dupmsite. It (Fe) were also detected in all the dumpsites as their concentrations were low. This can also be as a result of low Fe carrying waste. This is low compare to WHO standard .

FIGURE 1 Summarize the health impacts of heavy metals

4.10 MECHANISM OF REMEDIATING HEAVY METALS

Treatment process of acid mine water typically generate high density sludge that is heterogeneous due to variety of metals metalloid and anion components and this makes it difficult to dispose the sludge, present researches have therefore focused on the recovery of chemical species from acid mine drainage and secondary sludge, in order to protect human health, plant animal, soil and all the compartment of the environments, proper attention should be given to remediation technology of heavy metals.

FIGURE 2 HEAVY METAL REMEDIATION

CHAPTER FIVE

SUMMARY, CONCLUSION AND RECOMMENDATIONS

5.1 SUMMARY

The study work was based on the concentrations of heavy metal in soil refuse dump, plain clear field of Bajoga town, Gombe State Nigeria. The results indicate that some of the areas were shown to have the concentrations of heavy metals, though some heavy metals could not be detected in some soil refuge dump and plain clear field. It was observed that some soil refuge dump, plain clear field, have higher concentration of these metals (heavy metals) than others, and other site has low concentration of these heavy metals. This may be as a result of less contamination with those metals due to human activities. Some metals could not be detected; the implication is that the concentration of these metals are below detection limit or not present at all. These soil refuge dump and plain clear field have concentrations of heavy metals within the allowable limit; as a result, may not appear to pose very serious environmental problems at the moment. However, there is some concern as continuous accumulation in the levels of heavy metals may occur with time and may result in health challenges.

5.2 CONCLUSION

Base on the studies, some of the soil refuge dump in Bajoga town contained most of these heavy metals. It was also observed that some soil refuge dump had higher concentration of these metals than others. The heavy metals concentration in these soil refuge dump was all within the WHO acceptable limit in some soil refuge dump . Heavy metal concentration in soil refuse dump of Bajoga town may not appear to pose very serious environmental problems at the moment. However, accumulation of these metals may later pose a threat to human health and the environment.

5.3 RECOMMENDATIONS

The most effective way to reduce/eliminate the impact of these heavy metals on the environment is to develop and implement an effective waste management plan. Government should have established a better quality control unit for foods, crops to protect consumers from contamination exposure, in addition to close monitoring of the significant possibility of significant harm of these heavy metals on human health.

- Background concentration of heavy metals should be documented in different Environmental media around the world for later use as a reference point.

- The level of potential toxic heavy metals in sediments, soil and the resident biota should be assessed and monitored regularly.

- Regular surveys should be conducted to record the per capita daily consumption of fresh water and other food items such as rice by the resident population around the world. Such data will be valuable for more accurate and reliable human and ecological risk assessment.

- Efforts should be made to minimize heavy metal contamination in aquatic and terrestrial ecosystem to safeguard the biota and health of their consumers.

- the public should be educated about the harmful effects of toxic heavy metals on human health and the environments.

- Dumpsites should be treated before use especially for cultivation. Also the people living around these dumpsites should stop farming on or around them.

REFFERENCE

A. Ajayi and O. F. Kamson (1983): Determination of lead in   road side dust in Lagos city by atomic absorption spectrophotometer. International journal of environmental   issues Vol. 25. No. 7.

Adekunle, I.M Ndahi, N.P, & Owolabi, D.A (2003). Levels of Some Hazardous lead levels from Highway Soil, International Journal of Environmental Issues Development, University Consortia; Ikot-Ekpene, Nigeria 1 (1): 102

Connell, D.W., Miller. G.J (1984). Chemistry and Ecotoxicology of Pollution. JohnWiley & Sons, NY

Mark Tomlinson, Deepika Chaudhery Habibullah Ahmadzai, Sofia Rodriguez Gomez, Cecile Bizouerne, Thandi van Heyningen, & mickey Chopra (2020). International journal of mental health system

kristeen cherney2018.Medically reviewed by Elaine K. Luo, M.D

Michael S. marsh, MD, FRCOG 2008. Global women’s medicine king’s college london

Monisha Jaishankar, Tenzin Tseten, Naresh Anbalagan, Blessy B. Mathew, Krishnamurthy N. Beeregowda. Depertmen of		` Biotechnology, Sapthagiri College of engineering, Bangalore-57, Karnataka, Indian

Manavi Yadav,..Rakesh Kumar sharma (2019). In advances in purification technique

Nazanin Roohani, Richard Hurrell, Roya, Kelishadi, and Rainer Schulin (2013). Isfahan university of medical sciences, isfahan

Nyangababo, J.T. and Hamya, J.W. (1986). International. Environ.  Studies, 27: 115-9.

Wajahat Nazif, Sajida Perveen and Syed Asif Shah (2006). Evaluation of Irrigation Water for Heavy Metals of Akbarpura Area.Journal of Agricultural and Biological Science. VOL.1, NO.1: 51 -54

Vogels 1989

WHO/FAO/IAER. World health organisation. Switzerland: Geneva;1996 trace elements in human nutrition and health, (Akaeze C.S. 2001)